Last update: January 26, 2022

Geotextile Approved Products List for Adhesive Seams & Background Information

| Approved Product | Installation notes & Specification Requirements |
| --- | --- |
| - 3MTM Holdfast Cylinder Adhesive 70 | - Spray both sides of the fabric. - Six-inch minimum bonded overlap is required. - See packaging and Manufacturer for additional installation requirements and installation videos - See Manufacturer for Material Safety Data Sheet (MSDS) |

Additional Information: Seam performance is dependent on temperature variations, wet vs. dry material, adhesive application rate, dirty material at seam location, etc.  Pressure applied to the seam from a roller or other similar method is suggested to make a secure seam.  Operator skill and experience may contribute to the quality of the seam.  Test per the Schedule of Materials Control.

Background: Products will be reviewed and tested.  Only use adhesive on Types 3, 4, and 5 geotextiles.  Consult with the MnDOT Geotechnologies Engineer, Blake Nelson 651-366-5599 or Grading and Base Engineer, Terry Beaudry (651-366-5456) for special cases.